1. PROJECT OVERVIEW

JSC "Uzbekistan Airways" requests proposals for the repair and maintenance of two (2) LEAP-1A engines scheduled for shop visit during 2025-2026. This RFP outlines the technical requirements, commercial terms, and evaluation criteria for selecting qualified MRO providers.

Engine Serial Numbers:

ESN 59A632 (LEAP-1A33)

ESN 59A751 (LEAP-1A33)

2. SCOPE OF SERVICES

2.1 Aircraft and Engine Type

Aircraft: Airbus A320/A321neo

Engine Model: LEAP-1A (CFM International)

Workscope: Hot Section Refurbishment without LLP replacement

2.2 MRO Requirements

The selected participant must:

Possess EASA Part 145 certification for LEAP-1A engine maintenance

Have facilities, expertise, and experience in LEAP-1A engine repair

Comply with CFMI standards, procedures, and valid airworthiness authority regulations

Follow the agreed workscope and maintenance practices

2.3 Lease Engine Support (Preferred)

Participants with lease engine support capabilities will receive preferential consideration to minimize aircraft downtime.

3. TECHNICAL SPECIFICATIONS

3.1 Engine Details

| Parameter | ESN 59A632 | ESN 59A751 |
| --- | --- | --- |
| Engine Model | LEAP-1A26 | LEAP-1A33 |
| Time Since New | 8,678 FH / 2,711 FC | 14,108 FH / 3,172 FC |
| Time Since Last SV | First Run | First Run |
| LLP Limiter | 8,389 FC | 7,928 FC |
| Engine Condition | HPT Stage 1 Shroud Segment crack | HPT Stage 1 Shroud Segment crack |
| Workscope | Hot Section Refurbishment w/o LLP replacement | Hot Section Refurbishment w/o LLP replacement |

3.2 Performance Requirements

Minimum Engine Build Life: 9,000 Flight Cycles

EGT Margin Restoration: Target 70% of New Production pass-off result for Performance Restoration workscope

3.3 Maintenance Standards

Compliance: CFMI Maintenance Guide (latest revision)

Airworthiness Directives: All repetitive and open ADs must be incorporated

Service Bulletins: SBs requested by initial workscope must be embodied

Repairs Policy: PMA and DER repairs are not permitted

3.4 Special Requirements

HPT Blade Policy: HPT Blades with harsh environment operation history will not be accepted

Scrap Rate: 100% scrap rate expected for HPT Stage 1 Blades and HPT Stage 1 Nozzles

4. LOGISTICS AND TRANSPORTATION

4.1 Transportation Requirements

Outbound: DAP Incoterms 2010 (Tashkent to MRO facility)

Inbound: FCA Incoterms 2010 (MRO facility to Tashkent)

Compliance: OEM Engine Transportation Manual requirements

Insurance: Full coverage for transportation damage and carrier errors

4.2 Required Transportation Quote

Participants must provide separate pricing for:

One engine two-way transportation

Alternative transportation program offers (if applicable)

5. WARRANTY AND GUARANTEES

5.1 Standard Warranty Requirements

New OEM Parts: Standard manufacturer warranty handling

Workmanship Guarantee: 24 months / 5,000 FH for MRO workmanship, materials, and repairs

EGT Margin Guarantee: 70% of New Production pass-off result for performance restoration

5.2 Extended Warranty (Optional)

Participants may propose enhanced warranty programs for competitive advantage.

6. DELIVERABLES AND DOCUMENTATION

6.1 Post Shop Visit Documentation

The following documents must be provided upon completion:

EASA Form 1 Dual (EASA including FAA certificate)

Final Shop Finding Report

Updated LLP Status with full BTB history

Updated SB Status report

Updated QEC and Accessories Status report

Fan Blades mapping

Missing Parts list

Open Items List

List of replaced and scrapped parts

Certificates for all replaced parts

ETOPS 120 min Statement

Acceptance Test Report

BSI Report

Long Term Preservation tag

Engine shipment photographs including Data Plate

7. COMMERCIAL TERMS

7.1 Contract Type

Not-to-Exceed (NTE) or Capped Price non-exclusive contract with pricing stability requirements.

7.2 Turn Around Time (TAT)

TAT Start: First business day after engine delivery, workscope acceptance, and documentation acknowledgment

TAT End: Engine ready for redelivery in serviceable condition

Penalty: Daily credits for delays not attributed to excusable delays

7.4 Material Handling Rates

Participants must provide rates for:

New materials handling (% of CLP)

Used materials provision and handling (% of CLP)

New and used LLP handling (% of CLP)

Exchange fees (% of CLP)

Customer supplied parts handling (% of CLP)

Subcontracted services markup

Production, Engineering, and Extra Work labor rates

8. SUBCONTRACTING REQUIREMENTS

MRO providers may subcontract services to qualified EASA Part 145 organizations. Participants must provide forecasts for subcontracted engine parts requiring inspection, restoration, repair, or overhaul.

9. PROPOSAL SUBMISSION REQUIREMENTS

9.1 Technical Proposal

Company certifications and approvals

Facility capabilities and capacity

Experience with LEAP-1A engines

Proposed workscope methodology

Quality assurance procedures

TAT commitments and delay penalty structure

9.2 Commercial Proposal

Fixed pricing for each engine

Material handling rates

Transportation costs

Warranty terms and conditions

Payment terms acceptance

9.3 Additional Information

Lease engine support capabilities

Subcontracting strategy

Enhanced warranty offerings

Value-added services

10. EVALUATION CRITERIA

Proposals will be evaluated based on:

Technical Capability (30%)

Total Cost of Ownership (25%)

Turn Around Time (20%)

Warranty and Guarantees (15%)

Past Performance and References (10%)

11. SUBMISSION DEADLINE

Proposal Submission Deadline: [To be specified]

Questions Deadline: [To be specified]